GTI MAX DECOR CONNECT is a multi-layered interlocking looselay vinyl tile with polyurethane treatment (PUR+), 6 mm thick with a dovetail joint, abrasion group T, manufactured by Gerflor using a continuoushigh pressure process.

The GTI MAX DECOR CONNECT wearlayer isreinforced by specific oblong fillers that offers an outstanding resistance to traffic.

GTI MAX DECOR CONNECT is reinforced with 2 glass fibre grids on either side of an intermediate pressed homogeneous layer. GTI’s backing is a 4 mm thick black pressed homogeneous sheet, its surface is a coloured and reinforced homogeneous sheet 2 mm thick.

The product is 100% recyclable.